## EAT40005 Client Meeting Minutes

Date: April 24, 2025
Time: 4:30 PM – 5:30 PM
Location: AS324, Swinburne University
 Client: Alex Jaimes Castillo
 Group: Group 1 – Mobile App & 3D Model for LIDA Project
 Attendees:

- Alex Jaimes Castillo (Client)

- Senupama Deshapriya

- Haritha Manuja Perera

- Chrishelle Anthoneyz

- Thenura Kuruppuarachchi

Meeting Focus

Finalize details for the 3D animation of the LIDA composter and discuss sensor placement, structural improvements, and sequencing logic for animation and automation.

Key Discussion Points

🔧 Animation Components

- Motors: One per active and curing phase. Will rotate clockwise for 2 min and anticlockwise for 2 min. Frequency:

  - Active phase: 1–3 times/day

  - Curing phase: 1–2 times/week

- Animation Sequences:

  - Chamber opening & closing with motor stop logic for safety

  - Paddle mixing during active phase

  - Sequential gate opening after each composting phase

Sensor Locations

- Temperature sensors: Two each in compartments of active and curing phases

- Moisture sensors: Two in active, two in bottom section

- Gas sensor: Located near exhaust pipe to measure CO₂, CH₄, and O₂

- Motion sensors: Stop motor when chamber lid is opened

- Labeling: Sensors must be labeled with full names (e.g., Temp1) and optionally short codes (e.g., T1, MC1)

Design Modifications

- Top Lid:

  - Must be flat (not tilted)

  - Lid thickness: 2 cm for proper insulation

- Chambers:

  - Active and curing should be separate compartments

  - Bottom collection chamber must be simplified to 1 combined section with removable gate

- Exhaust and piping: Include clear pipe routing for gas outflow

Color Coding

- Red: Active phase

- Green: Curing phase

- Brown: Final compost area

Cycle Flow Planning

- Composting stages last 6 weeks total

  - Week 1–2: Active Phase

  - Week 3–4: Curing Phase

  - Week 5–6: Final collection

- Chamber opens every 2–3 days for waste addition; automated paddle mixing sequence follows

- Final animation will show full composting lifecycle

Mobile App Access

- Client provided a QR code to access the testing version of the app

- Team was asked to explore and understand the current interface and data readings

Client Feedback

- The animation logic is clear and acceptable

- Model improvements on lid fit and chamber insulation were highly encouraged

- Sensor labels should be clear and consistent

- Ready to receive updated model and animation preview by next meeting